Note :For work on ide arduino press on photo

*  RFID Module RC522

*  Simple Project:- Arduino will read RFID Tag and Display To the Serial Monitor!

*

*  RFID RC522 and Arduino Uno Pin Configuration

*

*  RFID RC522          Arduino Uno

*  SS/SDA              D10

*  SCK                 D13

*  MOSI                D11

*  MISO                D12

*  IRQ                 Not Connected

*  GND                 GND

*  RST                 D9

*  3.3V                3.3V

*

*  //Note: We connected LCD to the Analog Pins.

*  Remember, Analog Pins can be act as a Digital Pins,

*  But Digital Pins CAN NOT be act as a Analog Pins.

*

*  LCD(16x2) Connections

*

*  VSS pin to GND

*  VCC pin to 5V

*  //We are connecting "Contrast pin to GND for Maximum Brightness"

*  //You can also connect it to 10K Pot if you wish to change the Brightness Of LCD.

*  VEE pin to GND

*  RS pin to Analog pin A0

*  R/W pin to ground

*  Enable pin to Analog pin A1

*  D4 pin to Analog pin 5

*  D5 pin to Analog pin 4

*  D6 pin to Analog pin 3

*  D7 pin to Analog pin 2

*  LED+ pin to POWER

*  LED- pin to GND

*

*  Servo is Connected to Digital Pin 5

*

*  RGB LED

*  From RGB LED, We need Only Red and Green Color,

*  So we are not connecting Blue pin of the RGB.

*

*  Red pin to Digital pin 6

*  Green pin to Digital pin 7

*

*  Buzzer to Digital pin 8

*

*

*  Note:- RFID uses SPI Protocol to transfer the information.

*  I took the Melody(for the Buzzer) from,

*  File -> Examples -> Digital -> ToneMelody

*/

//Include sections

#include <RFID.h>

#include <SPI.h>

#include <LiquidCrystal.h>

#include <Servo.h>

#include "pitches.h"

//Define Component to Arduino Pins

#define SS_PIN 10

#define RST_PIN 9

#define SERVO_PIN 5

#define Red_LED 6

#define Green_LED 7

#define Buzzer 8

//initialize the library with the numbers of the interface pins

LiquidCrystal lcd(A0, A1, A2, A3, A4, A5);

Servo DoorLock;

RFID rfid(SS_PIN, RST_PIN );

//Unique ID of RFID Tag, which you want to give access.

int My_RFID_Tag[5] = {0x6a,0x10,0x41,0x3b,0x00};

//variable to hold your Access_card

boolean My_Card = false;

// notes in the melody, taken from:

//File -> Examples -> Digital -> ToneMelody

int melody[] = {

NOTE_C4, NOTE_G3, NOTE_G3, NOTE_A3, NOTE_G3, 0, NOTE_B3, NOTE_C4

};

// note durations: 4 = quarter note, 8 = eighth note, etc.:

int noteDurations[] = {

4, 8, 8, 4, 4, 4, 4, 4

};

void setup()

{

// put your setup code here, to run once:

//set the pins as an input/output

pinMode(Red_LED,OUTPUT);

pinMode(Green_LED,OUTPUT);

pinMode(Buzzer,OUTPUT);

//Servo Connnected to pin Digital Pin 5

DoorLock.attach(SERVO_PIN);

//open the serial port at 9600 baudrate.

Serial.begin(9600);

//Initialise the LCD to 16x2 Character Format

lcd.begin(16,2);

//Initialise Servo and RFID

SPI.begin();

rfid.init();

}

void loop()

{

// put your main code here, to run repeatedly:

//First Assume detected card(Or tag) is My_Card,

//Then later we will check is it My_Card or not!

My_Card = true;

DoorLock.write(0); //Servo at 0 Position, Door is Closed.

lcd.clear();

lcd.print("CTE COLLEGE");

lcd.setCursor(0,1);

lcd.print("GATE");

//Check if any RFID Tags Detected or not?

if( rfid.isCard() )

{

//if RFID Tag is detected, check for the Unique ID,

//and print it on the Serial Window

if( rfid.readCardSerial() )

{

lcd.clear();

lcd.print("UNIQUE ID is:- ");

delay(500);

lcd.setCursor(0,1); //Set LCD Cursor to Second Row, First Character

//Unique id is 5 Digit Number.

//Printing in HEX for better Understanding

for( int i = 0; i < 5; i++ )

{

Serial.print(rfid.serNum[i], HEX);

Serial.print(" ");

lcd.print(rfid.serNum[i],HEX);

lcd.print(" ");

}

delay(500);

//Compare this RFID Tag Unique ID with your My_RFID_Tag's Unique ID

for(int i = 0; i < 5; i++)

{

//if any one Unique ID Digit is not matching,

//then make My_Card = false and come out from loop

//No need to check all the digit!

if( My_RFID_Tag[i] != rfid.serNum[i] )

{

My_Card = false;

break;

}

}

Serial.println();

delay(1000);

//If RFID Tag is My_Card then give access to enter into room

//else dont open the door.

if(My_Card)

{

Serial.println("\CARD IS TRUE");

lcd.clear();

lcd.print("Welcome to Your");

lcd.setCursor(0,1);

lcd.print("GO");

delay(2000);

//Turn on the Green LED as an indication of permission is given

//to access the room.

digitalWrite(Green_LED,HIGH);

//Buzzer Config, taken from:

//File -> Examples -> Digital -> ToneMelody

// iterate over the notes of the melody:

int i = 0;

while(i < 2)

{

for (int thisNote = 0; thisNote < 12; thisNote++)

{

// to calculate the note duration, take one second

// divided by the note type.

//e.g. quarter note = 1000 / 4, eighth note = 1000/8, etc.

int noteDuration = 1000 / noteDurations[thisNote];

tone(8, melody[thisNote], noteDuration);

// to distinguish the notes, set a minimum time between them.

// the note's duration + 30% seems to work well:

int pauseBetweenNotes = noteDuration * 1.30;

delay(pauseBetweenNotes);

// stop the tone playing:

noTone(8);

}

i =  i + 1;

delay(500);

}

delay(1000);

//Now, Open the Door with the help of Servo Motor

DoorLock.write(180);

delay(200);

lcd.clear();

lcd.print("GATE is Open");

lcd.setCursor(0,1);

lcd.print("Now!");

delay(2000);

lcd.clear();

//Give 10 Sec delay to enter into room

//After that door will again closed!

for(int i = 10; i > 0; i--)

{

lcd.print("GATE  will close");

lcd.setCursor(0,1);

lcd.print("in ");

lcd.print(i);

lcd.print(" Sec.HurryUp!");

delay(1000);

lcd.clear();

}

//Now,Door is closed and Green LED is Turned-Off.

DoorLock.write(0);

digitalWrite(Green_LED,LOW);

delay(200);

lcd.clear();

lcd.print("GATE is Close");

lcd.setCursor(0,1);

lcd.print("Now!");

delay(2000);

}

// If RFID Tag is not My_Card then

// Do not open the Door and

//Turn-On Red LED and Buzzer as an indication of Warning:

//Somebody else is trying to enter into your room.

else

{

Serial.println("PLESE Get Out of Here !");

lcd.clear();

lcd.print("Card isNOT FOUND!");

lcd.setCursor(0,1);

lcd.print("PLESE Get OutHere!");

for(int i = 0; i < 7; i++)

{

digitalWrite(Buzzer, HIGH);

digitalWrite(Red_LED,HIGH);

delay(500);

digitalWrite(Buzzer, LOW);

digitalWrite(Red_LED,LOW);

delay(500);

}

delay(1000);

}

}

}

//Put RFID Reader into Halt, untill it not detects any RFID Tag.

rfid.halt();

}